| chapter 14 | - Fractional Distillation of Crude Oil Quiz Answer Key | BLM 14.4.3A |
| --- | --- | --- |
| - answer key |  |  |

bubble cap

gases

straight-run gasoline

naphtha

kerosene

gas oil

steam

residue

crude oil

fractionation tower

| Name of fraction | # of carbon atoms | Boiling point range (°C) | Common uses |
| --- | --- | --- | --- |
| Refining gases | 1–4 | below 32 | heating/cooling raw materials for chemical reactions |
| Motor vehicle gasoline | 5–9 | 32–104 | fuel |
| Naphtha | 10–12 | 104–157 | lighters, lab solvents |
| Kerosene | 12–14 | 157–232 | fuel |
| Gas oils | 15–18 | 232–426 | heating oils, diesel fuels |
| Heavy oils | 18 plus | 426 and higher | lubricating oil, grease, paraffin wax, asphalt |