.…BLM 8–5.…

Chapter 8 Practice Test

For questions 1 to 6, write the term from column B that completes the sentence in column A.

| A | B |
| --- | --- |
| 1. Different kinds of devices ___________________ different amounts of current. 2. Many buildings and most cars and trucks have a ___________________ box. 3. If a circuit is overloaded, the circuit breaker ___________________. 4. The arrangement of circuits in a building is often called the ___________________. 5. When you ___________________ an object, you connect it to Earth. 6. Current enters a building through a ___________________. | a) central electric panel b) draw c) fuse d) ground e) trips f) wiring |

7.	Decide whether each of the following statements is true or false. If it is false, rewrite it to make it true.

a)	True/False A device that can increase or decrease voltage is called a GFI outlet.

__________________________________________________________

b)	True/False A hot wire carries current away from a load.

__________________________________________________________

c)	True/False Most outlets in a home carry 120 V.

__________________________________________________________

d)	True/False A ground fault is any unsafe pathway for current to travel between a conductor and the ground.

__________________________________________________________

.…BLM 8–5.…

(continued)

8.	Complete the labels by naming each wire and its colour code.

9.	Under each image, write “safe” if the circuit shown is safe. Write “overloaded” if the circuit is overloaded. Show how you know.

- b)

_________________________________  __________________________

10.	Explain what will happen if a fourth 3 A bulb is connected in parallel to this circuit, and why.

____________________________________________________________

____________________________________________________________

11.	a)	Why would you use a GFI outlet?

_________________________________________________________

_________________________________________________________

b)	How does a GFI outlet help keep you safe around electricity?

_________________________________________________________

_________________________________________________________