.…BLM 8–6.…

Chapter 8 Test

For questions 1 to 6, write the term from column B that completes the sentence in column A.

| A | B |
| --- | --- |
| 1. If the current in a circuit is too high, the circuit is ___________________. 2. When an object is connected through a conductor to the earth it is ___________________. 3. A ___________________ contains the breakers and a directory of the circuits in a building. 4. A ___________________ is a safety device that has to be replaced when it burns out. 5. All homes are connected to a ___________________. 6. A ___________________ is a safety device that works like a switch. | a) power transmission grid b) fuse c) overloaded d) grounded e) breaker f) circuit panel |

7.	Decide whether each of the following statements is true or false. If it is false, rewrite it to make it true.

a)	True/False A circuit breaker has to be replaced when it trips.

__________________________________________________________

b)	True/False A ground wire is used to protect you from a ground fault.

__________________________________________________________

c)	True/False A neutral wire is white or red.

__________________________________________________________

d)	True/False A transformer can increase or decrease voltage.

__________________________________________________________

.…BLM 8–6.…

(continued)

8.	Complete the labels by naming each wire and its colour code.

9.	Under each image, write “safe” if the circuit shown is safe. Write “overloaded” if the circuit is overloaded. Show how you know.

- b)

_________________________________  __________________________

10.	Explain what will happen if a fourth 4 A bulb is
connected in parallel to this circuit, and why.

_______________________________________

_______________________________________

_______________________________________

11.	a)	You are going to put a new outlet over your bathroom sink. Why would a GFI outlet be a good choice?

__________________________________________________________

__________________________________________________________

b)	Explain how a GFI outlet works.

__________________________________________________________

__________________________________________________________