.…BLM 1–5.…

WHMIS Flash Cards

| WHMIS Symbol | What the Symbol Means | Risks | Precautions |
| --- | --- | --- | --- |
|  | Compressed Gas Normally stored at room temperature and kept in a pressurized container. Example: propane |  May explode because gas is under pressure.  May explode if heated or dropped.  May behave like a rocket if the cylinder is ruptured or if the valve is damaged. |  Secure cylinder to a wall or fixed object.  Keep cylinder away from heat.  Do not drop or allow to fall.  Do not stack objects on top of cylinders. |
|  | Oxidizing Material Material can cause other materials to burn or support combustion. Example: chlorine |  May burn skin or eyes if it touches them.  Increased fire and explosion hazard.  May cause flammable materials to explode or react violently. |  Wear body, hand, face, and eye protection.  Do not drop, puncture, or damage containers.  Store away from flammable materials in containers that will not rust. |
|  | Toxic Material that has harmful effects after long periods of time or after many exposures. Example: asbestos |  May cause death or permanent damage after repeated exposures.  May cause birth defects or sterility.  May cause cancer. |  Avoid breathing material by using respiratory protection.  Avoid direct contact by wearing proper body, hand, face, and eye protection.  Store only in designated areas. |

.…BLM 1–5.…

(continued)

|  | Corrosive Material reacts with metals and living tissue. Example: ammonia |  Can cause severe eye and skin irritation.  Can cause severe burns or tissue damage with long exposure.  Can cause lung damage if inhaled.  May cause blindness if touches eyes.  Fumes may cause environmental damage. |  Use breathing apparatus.  Work in well-ventilated areas.  Avoid all direct body contact.  Wear body, hand, face, and eye protection.  Use appropriate storage containers and ensure proper closures without vents. |
| --- | --- | --- | --- |
|  | Flammable Material continues to burn after being exposed to a flame or other ignition source. Example: gasoline |  May burst into flames spontaneously.  May release flammable products if allowed to break down or when exposed to water.  Burns easily. |  Work in well-ventilated areas.  Keep away from heat sources and sparks.  Do not smoke near these materials.  Store in a cool, fireproof area. |
|  | Poisonous Potentially fatal material that causes immediate and severe harm. Example: hydrogen cyanide |  May kill or cause permanent damage if eaten or breathed in, swallowed, or enters the body through skin or eye contact.  Small volumes have a toxic effect. |  Avoid breathing dust or vapours.  Work in well-ventilated areas and wear breathing protection.  Avoid contact with skin or eyes.  Wear face and eye protection.  Wear protective clothing. |

.…BLM 1–5.…

(continued)

|  | Biohazardous Infectious agent or biological toxin that can cause serious disease or death. Example: blood sample containing Hepatitis B virus |  May cause anaphylactic shock.  Includes viruses, yeasts, moulds, bacteria, and parasites.  Includes fluids containing toxic products. |  Special training is required.  Avoid breathing vapours.  Avoid contamination of people and other areas.  Store only in designated areas. |
| --- | --- | --- | --- |
|  | Reactive Material may have unexpected reactions. Example: benzoyl peroxide |  Is unstable.  May react with water.  May explode if exposed to shock or heat.  May release toxic or flammable vapours.  May burn unexpectedly. |  Handle with care avoiding vibration, shocks, and temperature changes.  Store in appropriate sealed containers in a cool flame-proof area. |