Chapter 7 Test

For #1 to #4, select the best answer.

1.	What is the angle of elevation 
of a cargo ramp that reaches 
a loading bay door 2 ft off the ground and extends 10 ft horizontally from the door?

A	2°	    B  11°    C  18°    D  78°

2.	During the demolition of a building, a support cable must 
be anchored to the floor 40 m from the base of a wall and attached 30 m up the side of wall. What is the angle of elevation of the cable?

A	37°    B  40°

C	46°    D  55°

3.	How long is a ladder whose base is 3 ft from a wall and makes an 80° angle with the ground?

A	10 ft    B  12 ft

C	14 ft    D  17 ft

4.	How tall is the building pictured below?

A	65 m      B  84 m

C	130 m    D  183 m

5.	A conveyor is being used to move shingles to a rooftop. The house is 8 m tall. The base of the conveyor is 15 m horizontally from the base of the house.

a)	Determine the angle of elevation needed to clear 
the rooftop of the house, 
to the nearest degree.

b) 	Determine how long the conveyor should be, to the nearest metre.

6.	A 1.5-m cable and a 1.2-m cable are staked in uneven ground 
0.8 m from the tree. The cables are connected to the trunk of 
the tree.

a)	Estimate the angle of elevation of each cable.

b)	Calculate the angle of elevation of each cable, to 
the nearest degree.

c)	How close were your estimates to your calculations? Why do you think this is?